Clarkson TCP/IP Packet Drivers under DESQview

A. Pirard - University of Liege - Belgium

This text explains technical details to be understood in order to

use the packet drivers under DESQview. I lists tested examples and

expresses hopes for even better TCP/IP usability under multitasking.

DESQview is a system that I've been using for some years and that

I find terribly useful. It does true multitasking and windowing of

normal DOS applications and adds keyboard macros and cut & paste

beautifully. The setup of some applications may require technical

knowledge or assistance, but once done correctly, they perform nicely.

In this text, 'application' means the set of DOS programs that are

executed in one window.

1 Theory

--------

When each DESQview application is installed, its maximum memory

size is indicated. DESQview itself can be installed in two flavors.

Either each application takes its size out of what DESQview does not

use of the 640 Kb the PC allows, and they must be swapped to disk to

make room for more. Or, with an EEMS or (true) LIM 4.0 board and

driver (expensive) or, by far the most comfortable, with a 386

processor and Quarterdeck's QEMM386 driver, much of the system

extensions, including DESQview, can be loaded above the 640 Kb limit,

and applications can execute simultaneously in large address spaces

(typically 535 KB) that can total up to physical (extended) memory

before swapping is required. The replicated address spaces are called

memory banks. The banks may be compared to the floors of a department

store, sharing the same ground area, but where different activities

can take place. DESQview 'switches in' each memory bank in turn to

give a few time slices of processor time to its application.

One problem under DESQview and EMM is that the packet drivers

execute calls to the TCP/IP applications. If they were executed before

DESQview is started, they would not be loaded in banked memory and

they would have to care to switch to the correct memory bank before

the call, like climbing to the right floor to call for the right

salesman.

Fortunately, there is a solution for the following two reasons:

1) These calls only occur only during a hardware interrupt. I

cannot swear for all packet drivers, but it cannot conceivably be

otherwise.

2) DESQview supervises hardware interrupts and detects when a

program grabs a vector to install a handler to service an interrupt

that DESQview considers for 'communication'. When such an interrupt

occurs, DESQview switches to the correct memory bank before giving

control to the handler. The 'communication' interrupts are: 2, 3, 4

and 7 (plus 5 on AT) and, in the latest versions (4.26), 8-15. (Note

that each application has its own set of interrupt vectors, but that

the handler will execute with the set of the application that was

interrupted, except its own interrupt 15h; it should rely only on its

private memory).

3) But when a Clarkson driver just interfaces with another

driver, the latter may in turn do calls to the first. In this case,

try to apply the same method as in the text about IBMTOKEN below.

So, if a packet driver is loaded as part of an application, its

initialization will grab the interrupt vector of the communication

card, DESQview will notice this, and interrupts and calls will occur

under the correct memory bank. Should it be loaded before DESQview is

active to see which interrupt is used, DESQview will not switch memory

during interrupts, and the calls it cannot control will be made to the

wrong memory bank and crash the system. Note that this doesn't hold

for a flat memory space (first flavor) because there is no memory

bank. But packet drivers are of no use to more than one application

simultaneously (as I will regret below), and it consumes less

permanent memory to load drivers in the application even then.

The application should close the device nicely by executing the

packet drivers utility 'termin'. Termin makes sure no more interrupts

will occur. Should termin not work (or not be given a chance to be

executed, when the window is closed abnormally), things may still

work. Here is why:

1) On a PC, the default interrupt handler should make sure they

don't bother the system (either nullifying service (EOI+IRET) or best

preventing them to occur (mask them at the controller)).

2) If the window is closed, DESQview returns the interrupts to

the default handler (that was used before DESQview was started).

However, I have experienced that a Token-Ring card will crash the

system when on interrupt IRQ2 but not on IRQ10 or IRQ11. Experiment if

necessary.

In any case, the application must be installed as non-swappable.

Note that the rumor that DESQview moves code about in memory is false,

only Windows does.

With QEMM, make sure it knows where your communication card RAM

is (QEMM parameters). Some cards hide the RAM until initialized.

2 Example

---------

A application using a packet driver can be installed by cloning

'Big DOS'. It must be non-swappable. Of course, 'Memory Size' depends

on storage available and needed by the TCP/IP program. The examples

below run with DESQview 2.26, QEMM 5.0 and drivers 7 on a PS/2 70:

Program Name............: CUTCP

Keys to Use on Open Menu: TR                         Memory Size (in K): 450

----------------------------------------------------------------------------

Program...: C:\TCPIP\tcpip

Parameters: call command /E:1024 /C c:\tcpip\cutcp tr g vm1

Directory.: C:\TEST

----------------------------------------------------------------------------

Options:

Writes text directly to screen.......: [Y]

Displays graphics information........: [Y]

Virtualize text/graphics (Y,N,T).....: [Y]

Uses serial ports (Y,N,1,2)..........: [Y]

Requires floppy diskette.............: [N]

System Memory (in K).......:   4   Maximum Program Memory Size (in K)..: 640

Script Buffer Size.......:  2000   Maximum Expanded Memory Size (in K):  256

Text Pages: 4  Graphics Pages: 2   Initial Mode:        Interrupts: 00 to FF

----------------------------------------------------------------------------

Window Position:

Maximum Height:  25       Starting Height:  25       Starting Row...:   0

Maximum Width.:  80       Starting Width.:  80       Starting Column:   0

----------------------------------------------------------------------------

Shared Program

Pathname..:

Data......:

----------------------------------------------------------------------------

Close on exit (Y,N,blank)......: [N]  Uses its own colors..............: [Y]

Allow Close Window command.....: [N]  Runs in background (Y,N,blank)...: [ ]

Uses math coprocessor..........: [Y]  Keyboard conflict (0-F)..........: [0]

Share CPU when foreground......: [Y]  Share EGA when foreground/zoomed.: [Y]

Can be swapped out (Y,N,blank).: [N]  Protection level (0-3)...........: [0]

with the procedure TCPIP:

c:\tcpip\drivers\3c523 0x60 7 0x300 0xC000

%1 %2 %3 %4 %5 %6 %7 %8 %9

c:\tcpip\drivers6\termin 0x60

exit

and the procedure CUTCP:

set $CUTCP1=include~c:\tcpip\cutcp.c%1tr (network administrator setup)

set $CUTCP2=include~c:\tcpip\cutcp.nik

c:\tcpip\cutcp\tn3270 -d 0 -h c:\tcpip\cutcp.tel %2 %3 %4 %5 %6 %7 %8

If one wishes DOS commands under packet drivers, use only the

application setup 'Parameter': 'call command /E:1024'. 'EXIT' will

terminate the application nicely.

Note: really, 3C523 v6.1 has (had?) a bug that can be overcome by

executing the version 5 once after each machine boot.

3 Token-Ring driver

-------------------

IBMTOKEN uses the IBM drivers DXMA0MOD.SYS and DXMC0MOD.SYS (A

and C for short). This complicates things a bit.

C is the driver to grab the communication vector and, for the

same reason as with packet drivers, must be loaded within the DESQview

application. But C calls A for services via interrupt 5C. And if

closing an application does not tell C to 'say good-bye' to A (if ever

there is provision for that) before simply removing it from memory, A

will tell C that it is already loaded when the application is started

again. Consequently, A too must be loaded inside the application so

that a fresh copy won't complain. But A sets flags in low core and

will say that it is already active. Resetting these flags is the end

of the story (almost, sorry but that's the way it is). DESQview

installs a front-end to make sure that interrupt 5C is not preempted

by another task. Letting things go would make A installing only its

own 5C processing, and without DESQview's front end, TCP/IP will lock

after a while by receiving no more IP data. Resetting the flag and

preserving DESQview's front-end is the trickery behind the modules

IBMTRDVx that I added to the Clarkson library.

Using IRQ2 for the Token-Ring card will crash the system when the

window is closed and C 'brutally' removed from memory. Configure it to

use IRQ10 or 11 instead (on a PS/2, that's booting with the reference

diskette to change the TRN card setup). Then, use the application

installation above with the following procedure TCPIP:

d:\ibmpclan\ibmtrdv1

device D:\IBMPCLAN\DXMA0MOD.SYS

d:\ibmpclan\ibmtrdv2

device D:\IBMPCLAN\DXMC0MOD.SYS ,

:or device D:\IBMPCLAN\DXMC0MOD.SYS 4000010430A0

c:\tcpip\drivers\ibmtoken 0x60 0

%1 %2 %3 %4 %5 %6 %7 %8 %9

c:\tcpip\drivers6\termin 0x60

d:\ibmpclan\ibmtrdv3

exit

Notes:

- the command 'device' is DESQview's, to load a SYS device driver as

if it were plain COM.

- Without the 'Locally managed address' parameter, a comma is

required. I don't know which of IBM or Quarterdeck is right about the

parameter memory format.

- The whole Token Ring stuff is my real best. It works fine. But I

cannot guarantee it does in all versions of the products involved.

4 Wishes

--------

Packet driver applications will execute in only a single DESQview

window, just like under monotasking DOS, but no worse. TCP/IP has been

devised with a multitasking environment in mind. MSDOS did not provide

multitasking, forcing people to write monotasking TCP/IP applications

instead. They probably simulate multitasking internally to some

extent. Now that we have DESQview and Windows 3, TCP/IP on the PC

should be rethought.

Multitasking DOS TCP/IP is making IP+TCP run in a separate window

and applications (in other windows) interface with them via a socket

API. This means an unloadable socket driver module that would depend

on the environment, and interface the applications with the module

containing IP+TCP. Under plain DOS, it would make plain calls. Under

DESQview or Windows, it would be in common memory and provide task

switching and synchronization (doing without loops).

TCP Inc have (maybe a standard for) a socket API and told me they

have tried but got problems to make it DESQview compatible. I guess it

should be feasible and that all is to be gained by everyone if FTP Inc

did cooperate with Quarterdeck and Microsoft for a tremendous

improvement of this API. I wouldn't be surprised most public domain

TCP/IP applications use the socket API, even if internally, and should

be easily converted to use an external one, maybe on a bimodal basis

to make them portable.

DESQview and Windows environments are similar enough in this

respect to kill the two birds with one stone.

I hope Quarterdeck won't mind my quoting their words. I'd like

them to be heard by FTP Inc. about feasibility:

"One thing I can tell you:  we are working on a driver which does

indeed watch for NETBIOS or IPX calls which would require that the

program be "mapped" back into memory in order for the call to operate

correctly."

And now that the packet drivers have promiscuous mode, a similar

enhancement would be most welcome at the data link level: recognize

the environment, remember the address space of subscribers and provide

task switching. Wouldn't it be lovely to have NETWATCH trace an

application in another window?.

Finally, I'll report some words about usability of TCP/IP

applications to us, those 'foreign' languages speakers. We badly need

an 8-bit character set. The problem is that extensions of ASCII on

different computers are incompatible and, for example, that text sent

from a PC to a Mac or vice versa is incorrect. Just as IBM EBCDIC has

to be translated to a common line code, ASCII, those different

extended codes should be translated to a common one. RFCs should state

(or programs act as if they did) that 8-bit is allowed (most often, 8-

bit 'works') and that the code to be used is the true international

standard ISO 8859. X-Windows already does that, why not Telnet, FTP

and SMTP to name the main ones? IBM shouldn't be allowed to slip in

its PC and EBCDIC codes by lack of standard. Even Telnet 3270 should

use ISO 8859 because no one but IBM mainframers know what EBCDIC is

(or should I say 'one' EBCDIC?).

But by far the easiest is that all those systems themselves start

using ISO 8859, isn't it?

Thanks for considering this point for us to enjoy TCP/IP as much

as you do.