Page 1

UTILITY NOTES:

(1)  PARK.COM

(2)  SCAT-SET.EXE

(3)  SCATEMM.SYS

(1)

NAME    : PARK.COM

PURPOSE : Park the hard disk head(s) before power off for safety.

SYNTAX  : PARK

COMMENTS: Hard disk heads are moved to disk's landing zone.

Page 2

(2)

NAME    : SCAT-SET.EXE

PURPOSE : Set the 80286 PC (with 82C235 chip-set) configuration.

Unless you are a professional engineer,we recommand that you

configure your system with the SCAT-SET utiliy.

Please refer to your User's Guide for detailed information.

SYNTAX  : SCAT-SET

Page 3

(3)

NAME    : SCATEMM.SYS

PURPOSE : A device driver to provide support for LIM EMS 4.0.

Please refer to the file "SCATEMM.TXT" for detailed information.

SYNTAX  : Example : (for reference only)

Copy the following command to your CONFIG.SYS.

DEVICE = SCATEMM.SYS

COMMENTS: The recommended procedure for installing SCATEMM.SYS is as follows:

<1>. Check to be sure that the system memory size is

at least 1M byte.

<2>. Type the command

DEVICE = SCATEMM.SYS

into your CONFIG.SYS file.

<3>. Run SCAT-SET --

a). Make sure the DRAM TYPE is correct.

b). Select 'EMS',select Enable EMS controller,

choose the desired EMS SIZE.

The EXTENDED MEMORY size will be adjusted automatically.

c). Save the current settings and exit SCAT-SET.

The system will reboot,with the EMS driver installed,

and EMS memory enabled.