Installation Documentation for Microsoft Word 5.5

=================================================================

The high resolution Microsoft Word 5.5 display driver supplied

with your SVGAXL VGA supports both high resolution text and

graphics. When using a Microsoft Mouse with the driver, please

ensure that your mouse driver version level is 7.0 or above.

Installing Word display drivers

To install the Word high resolution text driver supplied with

your SVGAXL, follow these instructions:

If you have not already done so, install Microsoft Word onto your

computer system. Follow the instructions supplied with Word. When

asked for a screen display type, choose IBM Video Graphics Array

(VGA).

Run the SVGAXL installation program (INSTALL.EXE), and choose the

option for the Microsoft Word drivers and specify the drive and

directory where you want them copied to (such as C:\WORD5).

Note: To use the new drivers they must be installed to the same

directory as Word. Otherwise, they must be copied to the Word

directory before they can be used.

INSTALL.EXE will copy two screen drivers:

SCREEN8.VID for 800x600 graphics resolution

SCREEN.VID - for 1024 x768 graphics resolution

The driver to be used must be named SCREEN.VID. Whan a file is

renamed this, it will overwrite the existing display driver. If

you wish to preserve the original, it should be renamed or copied

prior to installation of the new drivers.

The new driver is now installed, and to use it, run Word as you

normally would. To view and change the screen resolutions, select

Options, move the cursor to display mode, and press [F1]. Choose

the desired screen resolution from the list presented.

A sub-menu will appear, listing all the choices of resolutions

supported by the SVGAXL text driver. Select the desired

resolution.