Ref: PCJR Monitor Service (Combined three appends)

PCJR RGB Color Monitor Model 4863 was made by Mitsubishi Electric Corp.

and is identical sans case, speaker amp and PC cable to their

Model AT-1332A. The service manuel is available by writing:

Mitsubishi Elect. Corp.

991 Knox st.

Torrance, Ca. 90502

(213) 515-3993

For Parts:  Service Concept International

(714) 836-4981

Do not ask for a PCJR manual as they do not support IBM products.

The price is $25.00 for the complete manual,plus $4.00 ups ship charge.

And additional for C.O.D. if you don't prepay. (LIST/RETAIL may be

higher.)

They will sell copies of the schematic and parts list on request for

less, and send to you c.o.d..

The service manual you will get is a poor copy of a printed manual.

The schematic is barely readable but they claim it's the best they

have.

#1 most common problem with JR monitor is the following symptom:

Unit comes on with normal picture and operation and operates

for anywhere from a half hour to three hours with no problem

until a screen refresh occurs e.g. Alt-ctrl-del or some other

software momentary loss of sync signal to monitor.

The unit then loses horz sync with severe diagonal pulling of

the picture. No recovery is possible except turning off monitor

for a period of time. (Bare in mind, that when problem occurs,

no data will be lost if you simply leave the computer on.)

The simple solution is to remove the monitor cover and with a long

insulated shaft (tape it) thin blade screwdriver tweek the horz.

hold pot (VR502) to center the horz stability range. The pot is

located just to left of center (looking from front of set) and is

rather centrally located on the main base circuit board. It can be

reached from the top of the set straight down.

The adjustment will affect horz centering. If necessary, adjust

horz centering (three position switch at left front edge of board).

If correct centering still cannot be achieved then and only then

you may tweek the horz phase control (VR591) located about three

inches directly to the rear of VR502, and only for proper centering.

As a second method, from the Mitsubishi manual:

1. Receive a test pattern signal (Use JR diagnostics screen test)

2. Short TP-8A and TP-8B (don't know where they are)

3. Turn VR502 till picture is as stable as possible (it will roll

left to right or right to left very slowly.

4. Remove short.

#2 most common failure of JR monitor is the horz output transistor

Q502 type 2SD1426. Replace with ECG2302 available locally @$6.10.

Symptom is no picture/no sound from speaker/a high pitch squeel from

chassis/ smell after half a minute. Unplug unit--Q502 and R524 a

10 ohm 10 watt resistor will be too hot to touch.

Replace transistor to confirm repair.

If symptoms persist after replacing transistor, then most likely

the flyback transformer is shorted. P/N is 334P12101 (T503).

Although available from Mfg., cost and hassle to get it aren't worth

it.

#3 Most common problem is uncontrolable vertical rolling. This problem

has been traced to a defective C402 1.0 mfd @ 35v polarized tantalum

capacitor located off pin 10 of IC401, the vertical/horizontal

oscillator chip. It is in a rather difficult location to get at and in

order to solder a replacement you must remove the two bottom shields.

The metal shield is removed by unsoldering one ground wire and then

pushing up on the two plastic wire retaining posts at the front of

the monitor. The copper shield is removed by gently prying the three

plastic press in studs. Closing the barbs of these studs from the

component side helps greatly.

You should only use this info if you are qualified and have experiance

in servicing this type of equipment. CARELESSNESS CAN BE FATAL

The convertible RGB Color Monitor Model 5145 was made by Samsung of

Korea. I don't know of an equivalent model or whether a schematic

is available in the Convertible tech ref. manual.

The two types of monitors are 100% plug and function compatible. That

is either can be used with the PCJR or the PCC. Both have internal

speakers and a volume control, although the range and quality of

sound on the PCJR is much better due to use of an amplifier. The PCC

speaker is across the earphone jack and max volume is marginal.

Several previous appends have commented and I concur that the

resolution of the PCJR monitor is superior to the PCC monitor.

Both have slot mask picture tubes, but the beam width of the

red, blue and green guns of the PCC is too wide, or uneven between

the three colors causing an effect which appears as misconvergence

but in fact it is not correctable by convergence adjustments.

H Kennedy     Tie 678-6366    (HKENNEDY at AUSVM7)